# Statement of Work: Monoethylene Glycol (MEG) Transfer Pump Skid

Project Title: MEG Transfer Pump Skid

Project Number: BSC-2026-001

Client: B Star Chemetics

Location: Sarnia, Ontario, Canada

Project Type: Pump Station

Prepared By: A Star Chemical Engineering

Date: March 2026

## 1. Background

B Star Chemetics operates a glycol processing facility in Sarnia, Ontario. As part of a capacity expansion, a new pump skid is required to transfer monoethylene glycol (MEG) from an intermediate flash separator to the MEG regeneration column at elevated pressure.

## 2. Scope of Work

Design, procure, and commission a MEG transfer pump skid consisting of one (1) duty and one (1) standby horizontal centrifugal pump to transfer monoethylene glycol from the flash separator to the regeneration column.

The scope includes:

- Hydraulic design and pump sizing per 4-PRC-0007

- Pump and motor selection in accordance with API 610 (latest edition)

- Preparation of pump calculation sheet (4-CAL-0001)

- Preparation of pump data sheet (4-DST-0001)

- Process design basis and fluid property assessment per 1-PRC-0001

- Document register and project execution plan

## 3. Design Parameters

| Parameter | Value |
| --- | --- |
| Fluid | Monoethylene Glycol (MEG) |
| Fluid concentration | 95% by weight |
| Normal flow rate | 45 m³/h |
| Design pressure | 10 bar(g) |
| Operating temperature | 65 °C |
| Fluid density | 1,095 kg/m³ |
| Suction vessel pressure | 1.5 barg |
| Discharge vessel pressure | 3.2 barg |
| Suction static head | 3.0 m |
| Discharge static head | 8.0 m |
| Suction-side friction losses | 0.8 m |
| Discharge-side friction losses | 2.5 m |

## 4. Equipment

- Pump type: Horizontal centrifugal, API 610 OH2

- Configuration: 1 × duty + 1 × standby (100% installed spare)

- Driver: Electric motor, TEFC enclosure, 50 Hz

- Shaft seal: Mechanical seal, API Plan 11

## 5. Design Standards

- API 610 — Centrifugal Pumps for Petroleum, Petrochemical and Natural Gas Industries

- ASME B16.5 — Pipe Flanges and Flanged Fittings

- 4-PRC-0007 — Pump Calculation Procedure (AK Chemetics)

- 1-PRC-0001 — Process Design Basis (AK Chemetics)

- 1-LST-0002 — Fluid Properties List (AK Chemetics)

## 6. Deliverables

The following documents shall be produced as part of this project:

| Doc ID | Description | Discipline |
| --- | --- | --- |
| 0-REP-0001 | Project Execution Plan | PM |
| 0-SCH-0001 | Project Schedule | PM |
| 1-CAL-0001 | Process Hydraulic Calculation | Process Engineer |
| 4-CAL-0001 | Pump Hydraulic Calculation | Mechanical Engineer |
| 4-DST-0001 | Pump Data Sheet | Mechanical Engineer |

## 7. Project Schedule

| Milestone | Target Date |
| --- | --- |
| Project Kickoff | April 2026 |
| Process Design Basis Approved | April 2026 |
| Pump Calculation Issued for Review | May 2026 |
| Pump Data Sheet Issued for Enquiry | June 2026 |
| Vendor Quotations Received | July 2026 |
| Purchase Order Placed | August 2026 |
| Project Complete | October 2026 |

## 8. Constraints and Assumptions

- The MEG flash separator and regeneration column nozzle sizes and elevations are fixed.

- Piping isometrics and detailed friction loss calculations to be finalized at detailed design stage; preliminary friction losses are as stated in Section 3.

- Pump vendor to confirm NPSHr; preliminary NPSHr estimate used for initial sizing.

- Motor voltage: 480 V, 3-phase, 60 Hz.

- Ambient temperature: 20 °C (design basis for motor rating).