### Mock SOW: Rio Tinto H2SO4 Storage & Loading Facility

1. Project Overview This Scope of Work (SOW) involves the engineering and procurement of two (2) horizontal centrifugal pumps for the sulfuric acid storage terminal at the Rio Tinto facility in Gladstone, Australia. The pumps are required to unload 98% Concentrated Sulfuric Acid (H2SO4) from rail cars to storage tanks.

2. Technical Specifications

- Equipment Type: Horizontal Centrifugal Pump

- Fluid Handled: 98% Sulphuric Acid (H2SO4)

- Design Flow Rate: 85 m³/h

- Design Pressure: 12 bar

- Design Temperature: 45°C

- Suction Pressure: 0.8 barg

- Discharge Pressure: 7.5 barg

- Fluid Density: 1,840 kg/m³ (Note: High density)

- Viscosity: 25 cP (at 45°C)

3. Engineering Requirements & Risks

- Material Selection: Due to the extreme reactivity of 98% H2SO4, material selection is critical. The pump must be specified with alloy materials (e.g., Alloy 20 or similar) capable of handling concentrated acid. Cross-reference with 5-LST-0003 for approved materials.

- Safety & Sealing: A double mechanical seal with an external barrier fluid (API Plan 52 or 53) is mandatory to prevent any acid leakage to the environment.

- Compliance: Documentation must strictly follow the 0-PRC-0002 numbering system and the 1-PRC-0001 process design criteria.

4. Expected Deliverables The AI Agent must provide:

- An updated Document Register including a specific "Materials Selection Report".

- Hydraulic calculations considering the high fluid density (SG = 1.84) and its impact on motor power and TDH.

- A clear identification of the fluid code from 1-LST-0002.