| Rev: | Date: | Status: | Prepared By: | Checked By: | Approved By: |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 0 |  |  |  |  |  |

# Purpose

This specification defines the general requirements for the design, fabrication and supply of horizontal centrifugal pumps

# Scope

The Vendor will supply the equipment fully assembled with all items required for a complete functional unit including the following major components:

- Pump wet end and power end

- Mechanical seal complete with seal guard

- Complete coupling complete with coupling guard

- [Motor]

- Baseplate

# Process Requirements

The pumps will meet the process requirements as detailed in the individual Pump Data Sheets.  If a Pump Data Sheet states a flow rate value + ‘Vendor Minimum Continuous Flow’ (MCF) the Vendor will state their recommended MCF rate for the pump they are selecting, and add this MCF to the flow rate value given on the Data Sheet.  For example, if the Pump Data Sheet states a 10 m3/h + ‘Vendor Minimum Continuous Flow’, and the recommended MCF rate for the pump being selected is 1 m3/h, then the duty point will become 11 m3/h.

The pumps will have stable head/capacity curves that continuously rise to shut-off.

The pumps will not have a design duty point that is to the right of the head/capacity curve Best Efficiency Point unless approved by Aker Kvaerner Chemetics.

# Mechanical Requirements

The pumps will be designed in accordance with [ASME B73.1: Specification for Horizontal End Suction Centrifugal Pump for Chemical Process].

The Equipment will be dynamically balanced in accordance with ISO1940: Mechanical vibration - Balance quality requirements of rigid rotors - grade [G6.3].

The Vendor will provide the estimated noise level of the pumps.

The selected impeller diameter will neither be greater than 90% of the maximum size available for the selected pump casing nor less than 110% of the minimum size without approval from Aker Kvaerner Chemetics.  Once a pump casing has been selected, the Vendor will proceed with the maximum size impeller for the casing, and will not trim the impeller until after the final size is confirmed by Aker Kvaerner Chemetics, in writing.

The maximum operating speed will be less than the synchronous speed of a two-pole motor.

The pump suction and discharge connections will be flat faced flanged and will match the [DIN PN10] standard.  A casing drain will be provided with [an NPT] plug.  The seal fluid connection will be provided with a temporary [NPT] plug.

Baseplates and couplings will be large enough to accept motors one size larger than the size selected.  Spacer blocks will be provided to allow installation of the larger motor without affecting pump process connections.  Baseplates will be [polymer concrete].

Impellers will be semi-open unless noted otherwise on the individual Pump Data Sheets

The pump seal chamber bores will be enlarged and tapered.

[Others] will supply the seal fluid control systems.  The Vendor will specify the seal fluid pressure and flow requirements.

Motors will be sized for the run-out condition at the selected impeller size.

General equipment requirements will be per 4-SPC-0001: General Equipment Requirements.

Commentary

To Section 1.0: A specification is used to:

- Briefly and clearly communicate the main requirements to the supplier

- Allow easy addition of special contract requirements (if applicable)

- Minimize editing of data sheets

- Allow data sheets to more clearly show special cases by exception

To Name Change Conversion: Name change has been implemented. No new revision number has been used and changes have not been marked.

To 2005 Name Change Conversion:  Name change has been implemented.  No new revision number has been used and changes have not been marked.