## PUMP DATA SHEET

Document No.: 4-PDS-XXXX

Revision: 0

Date: 2023-10-27

Prepared By: [Your Name/Department]

PROJECT INFORMATION

Client: B Star Chemetics

Project: MEG Transfer Pump Skid

Location: Sarnia, Ontario, Canada

Equipment Tag: P-001 A/B

### 1. EQUIPMENT IDENTIFICATION

| Field | Value |
| --- | --- |
| Equipment Tag | P-001 A/B |
| Service | Monoethylene Glycol (MEG) Transfer |
| Quantity | 2 (1 Duty + 1 Standby) |
| Pump Type | Centrifugal (Radial Flow) |
| Application | Process Transfer |

### 2. OPERATING CONDITIONS

| Field | Value | Unit | Notes |
| --- | --- | --- | --- |
| Fluid | Monoethylene Glycol (MEG), 95% wt | - | Code: MEG |
| Density | 1095 | kg/m³ | SG: 1.095 |
| Viscosity | 4 | mPa·s |  |
| Vapour Pressure | 0.5 | kPa | At 65°C |
| Corrosive | True | - | Requires careful material selection |
| Rated Flow | 54 | m³/h |  |
| Total Dynamic Head | 24.17 | m |  |
| Design Pressure | 10 | bar | PN10 Flange Rating |
| Design Temperature | 65 | °C |  |
| NPSHa | 25.6 | m | Net Positive Suction Head Available |

### 3. HYDRAULIC CALCULATION SUMMARY

(Calculations performed as per 4-PRC-0007 / Pump_Calculation_Template)

| Field | Value | Unit | Notes |
| --- | --- | --- | --- |
| Total Suction Head | 25.55 | m |  |
| Total Discharge Head | 49.72 | m |  |
| Total Dynamic Head (TDH) | 24.17 | m | (Discharge Head - Suction Head) |
| Hydraulic Power | 3.89 | kW |  |
| Pump Efficiency (η) | 0.72 | - | Estimated |
| Shaft Power (BHP) | 5.4 | kW | (Hydraulic Power / η) |
| Motor Sizing Margin | 1.25 | - | As per API 610 |
| Required Motor Power | 6.75 | kW | (Shaft Power × Margin) |
| Specific Speed | 16.3 | - | Indicates Radial Flow Pump |
| NPSHr Estimate | 4.0 | m | Net Positive Suction Head Required (Vendor to confirm) |
| NPSHa vs NPSHr Status | PASS | - | (25.6 m NPSHa > 4.0 m NPSHr) |

### 4. MATERIALS OF CONSTRUCTION

(Referenced Standards: 4-SPC-0001, 4-SPC-0002, 5-LST-0003)

| Part Group | Component | Material Specification | Justification / Notes |
| --- | --- | --- | --- |
| ------------------ | **-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------- |  |  |

MEG Transfer Pump Data Sheet

Client: B Star Chemetics

Project: MEG Transfer Pump Skid

Location: Sarnia, Ontario, Canada

Document No.: 4-PDS-XXXX

Revision: 0

Date: 2023-10-27

Prepared By: [Mechanical Engineer's Name/Department]

### 1. EQUIPMENT IDENTIFICATION

| Field | Value | Remarks |
| --- | --- | --- |
| Equipment Tag | P-001 A/B | Primary (A) and Standby (B) Pump |
| Service | Monoethylene Glycol (MEG) Transfer | Continuous duty process transfer |
| Quantity | 2 | (1 Duty + 1 Standby) |
| Pump Type | Centrifugal | Radial Flow (based on Specific Speed) |
| API Compliance | API 610 (latest edition) | For heavy-duty petrochemical service |

### 2. OPERATING CONDITIONS

| Field | Value | Unit | Source / Remarks |
| --- | --- | --- | --- |
| Fluid | Monoethylene Glycol (MEG), 95% wt | - | From Process Engineer (Code: MEG) |
| Density | 1095 | kg/m³ | From Process Engineer (SG: 1.095) |
| Viscosity | 4 | mPa·s | From Process Engineer |
| Vapour Pressure | 0.5 | kPa | At 65°C (From Process Engineer) |
| Corrosive | True | - | Requires specific SS grade selection |
| Rated Flow | 54 | m³/h | Design Basis |
| Total Dynamic Head | 24.17 | m | Design Basis |
| Design Pressure | 10 | bar | PN10 Flange Rating (Suction/Discharge) |
| Design Temperature | 65 | °C |  |
| NPSHa | 25.6 | m | Net Positive Suction Head Available |

### 3. HYDRAULIC CALCULATION SUMMARY

(Calculations performed as per B Star Chemetics Standard 4-PRC-0007, using Pump_Calculation_Template)

| Field | Value | Unit | Calculation / Remarks |
| --- | --- | --- | --- |
| * Wetted Parts \\* |  |  |  |
| Casing | Stainless Steel | To be confirmed by Vendor | Compatible with 95% MEG at 65°C as per 4-SPC-0001 and 5-LST-0003. Vendor to propose specific grade for approval. |
| Impeller | Stainless Steel | To be confirmed by Vendor | Compatible with 95% MEG at 65°C as per 4-SPC-0001 and 5-LST-0003. Vendor to propose specific grade for approval. |
| Shaft | Stainless Steel | To be confirmed by Vendor | Compatible with 95% MEG at 65°C as per 4-SPC-0001 and 5-LST-0003. Vendor to propose specific grade for approval. |
| \\Non-Wetted Parts\\ |  |  |  |
| Baseplate | Carbon Steel | Painted/Coated | As per B Star Chemetics Standard 4-SPC-0002 for protective coating. |
| \\Special Requirements\\ |  |  |  |
| Corrosive Service | Specific Stainless Steel Grade | Vendor Proposal Required | Due to corrosive nature of MEG, specific SS grade (e.g., 316L, Duplex) must be proposed by vendor and approved by B Star Chemetics. |
| Ambiguous Reference | 'Seat' in [8-LST-0112] | Clarification Required | Vendor to investigate and clarify the reference to 'Seat' in [8-LST-0112] as it is ambiguous for general wetted parts. Confirm if this refers to specific wear parts or general wetted components. |

### 5. MECHANICAL SEAL SPECIFICATION

| Field | Value | Remarks |
| --- | --- | --- |
| Seal Type | Double Mechanical Seal | Required for critical and corrosive service |
| API Plan | API Plan 53B | Pressurized barrier fluid system |
| Barrier Fluid | Required | Vendor to specify fluid and system details |
| Barrier System | Vendor Supplied | Includes reservoir, cooler, instrumentation, piping. Must meet API 682 requirements. |
| Seal Face Mat. | To be confirmed by Vendor | Compatible with MEG and barrier fluid |
| Elastomers | To be confirmed by Vendor | Compatible with MEG and barrier fluid |

### 6. VENDOR REQUIREMENTS

6.1. Vendor Confirmation:

The Vendor shall confirm the following:

Pump performance (TDH, flow, efficiency, NPSHr) at rated conditions.

Specific material grades for all wetted parts (Casing, Impeller, Shaft) compatible with 95% MEG at 65°C, referencing B Star Chemetics Standard 4-SPC-0001 and 5-LST-0003.

Motor sizing and compliance with API 610 margin (1.25x BHP).

Mechanical seal arrangement and barrier fluid system (API Plan 53B) details, including barrier fluid type, pressure, and temperature.

Compliance with API 610 (latest edition) for centrifugal pumps in petrochemical service.

6.2. Vendor Submittals (Minimum Requirements):

The Vendor shall submit the following documents for review and approval:

Certified pump performance curves (Head vs. Flow, Efficiency vs. Flow, BHP vs. Flow, NPSHr vs. Flow).

General Arrangement (GA) drawings of the pump and motor assembly, including dimensions, weights, and nozzle orientations.

Detailed Material Specification List (MDS) for all pump components, including specific grades of stainless steel for wetted parts.

Mechanical seal drawings, data sheets, and P&ID for the API Plan 53B barrier fluid system.

Motor data sheet, performance curves, and certified drawings.

Coupling data sheet and drawings.

Baseplate drawings.

Sound level data at rated conditions.

Vibration analysis report.

Quality Control Plan (QCP) and Inspection and Test Plan (ITP).

6.3. Testing Requirements:

The Vendor shall perform the following tests:

Hydrostatic Test: Casing hydrostatic test as per API 610.

Performance Test: Full performance test (flow, head, power, efficiency, NPSHr) at rated conditions and across the operating range as per API 610.

Vibration Test: Vibration measurements as per API 610.

Witnessed Tests: All major tests (hydrostatic, performance, vibration) shall be witnessed by B Star Chemetics or their appointed third-party inspector, as per B Star Chemetics Standard 4-SPC-0002.

NPSHr Test: NPSHr curve verification.

### 7. OPEN ITEMS

The following items are TBD or require vendor data/clarification to finalize:

Specific Stainless Steel Grades: Vendor to propose specific grades for Casing, Impeller, and Shaft for approval by B Star Chemetics, ensuring compatibility with 95% MEG at 65°C and compliance with 4-SPC-0001 and 5-LST-0003.

Barrier Fluid System Details: Vendor to provide detailed specifications for the barrier fluid (type, properties) and all associated API Plan 53B system components (e.g., reservoir volume, cooler type/capacity, instrumentation, piping materials).

'Seat' Reference Clarification: Vendor to clarify the ambiguous reference to 'Seat' in [8-LST-0112] regarding its application to general wetted parts or specific wear components.

Final Pump Model Selection: Specific pump model and manufacturer to be confirmed based on vendor proposals and technical evaluation.

Coupling & Guard: Type of coupling and guard to be specified by vendor and approved by B Star Chemetics.

Instrumentation: Detailed instrumentation requirements (e.g., pressure gauges, temperature indicators, vibration monitors, flow meters) for the pump and seal system will be specified in a separate P&ID and Instrument Data Sheets.

Motor Enclosure & Voltage: To be specified in a separate Motor Data Sheet.

Flange Ratings: Confirm specific flange ratings (e.g., ANSI Class 150/300 or PN10/16) for suction and discharge nozzles. (Currently PN10 specified for design pressure).

END OF DATA SHEET